ONOS-ISIS MANUAL

- Download ONOS on your System.

- Refer link: https://wiki.onosproject.org/display/ONOS/ONOS+for+Newcomers

- Run the Maven Build for ONOS using the following command.

- mvn clean install

- Start onos-karaf using the command.

- onos-karaf clean install

- Download the C Program as per the instruction given in the ISIS Wiki page

https://wiki.onosproject.org/display/ONOS/ISIS+as+SBI

- Go to the directory where C Program is saved.

- Compile the program using the make file

- Make debug

- Run isis_ospf

- sudo ./isis_ospf isis

- Use the isis-config Json file given below.

- Activate ISIS using the command on the ONOS-console.

- app activate org.onosproject.isis

- Download and Configure the Rest Client.

- Select post mode under method tab

- Select string body under the body tab

- Select Application/Json under it

- Browse the Json file from the stored location

- Select the authentication as basic under Auth tab.

- Give the user name and password as karaf

- Click on run

- Use wire shark to observe the packets on the network.

- Use the following Cli command to view isis database on ONOS-console.

- isis database process-id

- Use the following Cli command to view isis Neighbor’s on ONOS-console

- isis neighbor process-id

- Use the following Cli command to view isis Neighbor’s on ONOS-console

  - isis interface process-id

- Use the command to de-activate isis on onos.

- app deactivate org.onosproject. isis

## Sample Json file for one interface

{

"apps": {

"org.onosproject.provider.isis.cfg": {

"isisapp": {

"method": "add",

"processes": [{

"processId": "4.4.4.4",

"interface": [{

"interfaceIndex": "2",

"intermediateSystemName":"ROUTERONE",

"systemId": "2929.2929.2929",

"reservedPacketCircuitType": "2",

"circuitId": "10",

"networkType": "2",

"areaAddress": "490001",

"holdingTime": "50",

"helloInterval": "10"

}]

}]

}

}

}

}

## Sample json for multiple interface

{

"apps": {

"org.onosproject.provider.isis.cfg": {

"isisapp": {

"method": "add",

"processes": [{

"processId": "4.4.4.4",

"interface": [{

"interfaceIndex": "2",

"intermediateSystemName": "ROUTERONE",

"systemId": "2929.2929.2929",

"reservedPacketCircuitType": "2",

"circuitId": "10",

"networkType": "2",

"areaAddress": "490001",

"holdingTime": "50",

"helloInterval": "10"

},

{

"interfaceIndex": "3",

"intermediateSystemName": "ROUTERTWO",

"systemId": "3939.3939.3939",

"reservedPacketCircuitType": "2",

"circuitId": "100",

"networkType": "2",

"areaAddress": "490001",

"holdingTime": "50",

"helloInterval": "10"

}]

}]

}

}

}

}

- There can be multiple interface under one process ID.

- The process ID can be of any format.

- The area ID can be of any format.

- The Reserved Packet Circuit Type field should be 1 for Level 1, 2 for level 2 and 3 Level 1&2.

- In order to set the network type give the value of interface type as 1 for P2P and 2 for Broadcast.

- Each interface should have a unique interface index.