ONOS-OSPF MANUAL

- Download ONOS on your System.

- Refer link: https://wiki.onosproject.org/display/ONOS/ONOS+for+Newcomers

- Run the Maven Build for ONOS using the following command.

- mvn clean install

- Start onos-karaf using the command.

- onos-karaf clean install

- Download the C Program as per the instruction given in the OSPF Wiki page

https://wiki.onosproject.org/display/ONOS/OSPF+as+SBI

- Go to the directory where C Program is saved.

- Compile the program using the make file

Make debug

- Run isis_ospf.

sudo ./isis_ospf ospf

- Use the ospf-config Json file given below.

- Download and Configure the Rest Client.

- Select post mode under method tab

- Select string body under the body tab

- Select Application/Json under it

- Browse the Json file from the stored location

- Select the authentication as basic under Auth tab.

- Give the user name and password as karaf

- Click on run

- Activate OSPF using the command on the ONOS-console.

- app activate org.onosproject.ospf

- Use wire shark to observe the packets on the network.

- Use the following Cli command to view ospf database on ONOS-console

- Show ospf databse process-id

- Use the following Cli command to view ospf Neighbor’s on ONOS-console

- Show ospf neighbors process-id

- Use the command to de-activate ospf on onos.

- app deactivate org.onosproject.ospf

## Sample Json file for one interface

{

"apps": {

"org.onosproject.provider.ospf.cfg": {

"ospfapp": {

"method": "add",

"processes": [{

"processId": "4.4.4.4",

"areas": [{

"areaId": "5.5.5.5",

"routerId": "7.7.7.7",

"isOpaqueEnable": "false",

"externalRoutingCapability": "true",

"interface": [{

"interfaceIndex": "2",

"helloIntervalTime": "10",

"routerDeadIntervalTime": "40",

"interfaceType": "2",

"reTransmitInterval": "5"

}]

}]

}]

}

}

}

}

## Sample json for multiple interface

{

"apps": {

"org.onosproject.provider.ospf.cfg": {

"ospfapp": {

"method": "add",

"processes": [{

"processId": "4.4.4.4",

"areas": [{

"areaId": "5.5.5.5",

"routerId": "7.7.7.7",

"isOpaqueEnable": "false",

"externalRoutingCapability": "true",

"interface": [{

"interfaceIndex": "2",

"helloIntervalTime": "10",

"routerDeadIntervalTime": "40",

"interfaceType": "2",

"reTransmitInterval": "5"

},

{

"interfaceIndex": "3",

"helloIntervalTime": "10",

"routerDeadIntervalTime": "40",

"interfaceType": "2",

"reTransmitInterval": "5"

}]

}]

}]

}

}

}

}

- There can be multiple areas under one process ID.

- There can be multiple interfaces declared under one single area.

- The process ID can be of any format.

- The area ID can be of any format.

- The Router ID should be in an IP address format.

- ISOPaqueEnabled field should be either true of false.

- In order to set the network type give the value of interface type as 1 for P2P and 2 for Broadcast.

- Each interface should have a unique Interface Index